Materials and coding used

- PVC Pipe (used as place holder for our cane)

- Box (as a case for our components)

- Bread board

- Jumper Wires

- Button

- Buzzer

- Ultrasonic Sensor

- Rain Detector

Microcontrollers

- MPU 6050

- Arduino

Coding

#include <WiFi.h>

#include <WiFiClientSecure.h>

#include <Wire.h>

#include <MPU6050.h>

#include <UniversalTelegramBot.h>

#include <ArduinoJson.h>

// ================= WIFI =================

const char* ssid = "zzz";

const char* password = "12345627";

// ================= TELEGRAM =================

#define BOT_TOKEN "8317803239:AAHHJ5fQ3zo6IAuJKGbZcFwXNiZ8hOgSYP0"

#define CHAT_ID   "1045076189"

WiFiClientSecure client;

UniversalTelegramBot bot(BOT_TOKEN, client);

// ================= MPU6050 =================

MPU6050 mpu;

// ================= CALIBRATION =================

float pitchOffset = 0;

float rollOffset  = 0;

bool calibrated = false;

// ================= FALL DETECTION =================

float ax, ay, az;

float pitch, roll;

float pitchCorrected, rollCorrected;

const float ANGLE_THRESHOLD = 45.0;     // degrees

const unsigned long FALL_TIME = 2000;   // must stay tilted (ms)

const unsigned long ALERT_COOLDOWN = 30000;

unsigned long tiltStartTime = 0;

unsigned long lastAlertTime = 0;

bool tiltDetected = false;

// ================= CALIBRATION FUNCTION =================

void calibrateOrigin() {

int16_t rawAx, rawAy, rawAz;

float pitchSum = 0, rollSum = 0;

const int samples = 100;

Serial.println("📐 Calibrating... Keep device upright");

for (int i = 0; i < samples; i++) {

mpu.getAcceleration(&rawAx, &rawAy, &rawAz);

float x = rawAx / 16384.0;

float y = rawAy / 16384.0;

float z = rawAz / 16384.0;

float p = atan2(x, sqrt(y*y + z*z)) * 180 / PI;

float r = atan2(y, sqrt(x*x + z*z)) * 180 / PI;

pitchSum += p;

rollSum  += r;

delay(10);

}

pitchOffset = pitchSum / samples;

rollOffset  = rollSum  / samples;

calibrated = true;

Serial.println("✅ Calibration complete");

Serial.print("Pitch offset: ");

Serial.println(pitchOffset);

Serial.print("Roll offset: ");

Serial.println(rollOffset);

}

// ================= SETUP =================

void setup() {

Serial.begin(115200);

Wire.begin(21, 22);

mpu.initialize();

if (!mpu.testConnection()) {

Serial.println("❌ MPU6050 not detected");

while (1);

}

Serial.println("✅ MPU6050 connected");

delay(3000);        // time to place device upright

calibrateOrigin();  // set origin angle

client.setInsecure();

WiFi.begin(ssid, password);

Serial.print("Connecting to WiFi");

while (WiFi.status() != WL_CONNECTED) {

delay(500);

Serial.print(".");

}

Serial.println("\n✅ WiFi connected");

bot.sendMessage(CHAT_ID,

"🟢 ESP32 Fall Detection ONLINE\n📐 Angle calibrated", "");

}

// ================= LOOP =================

void loop() {

int16_t rawAx, rawAy, rawAz;

mpu.getAcceleration(&rawAx, &rawAy, &rawAz);

ax = rawAx / 16384.0;

ay = rawAy / 16384.0;

az = rawAz / 16384.0;

pitch = atan2(ax, sqrt(ay*ay + az*az)) * 180 / PI;

roll  = atan2(ay, sqrt(ax*ax + az*az)) * 180 / PI;

pitchCorrected = pitch - pitchOffset;

rollCorrected  = roll  - rollOffset;

Serial.print("Pitch: ");

Serial.print(pitchCorrected);

Serial.print(" | Roll: ");

Serial.println(rollCorrected);

unsigned long now = millis();

// ===== FALL LOGIC =====

if (abs(pitchCorrected) > ANGLE_THRESHOLD ||

abs(rollCorrected)  > ANGLE_THRESHOLD) {

if (!tiltDetected) {

tiltDetected = true;

tiltStartTime = now;

Serial.println("⚠️ Tilt detected");

} else if (now - tiltStartTime >= FALL_TIME &&

now - lastAlertTime > ALERT_COOLDOWN) {

sendFallAlert();

lastAlertTime = now;

tiltDetected = false;

}

} else {

tiltDetected = false;

}

delay(50);

}

// ================= TELEGRAM ALERT =================

void sendFallAlert() {

Serial.println("🚨 FALL DETECTED");

String message;

message  = "🚨 *FALL DETECTED!*\n";

message += "Abnormal tilt detected.\n";

message += "User may need assistance.";

bot.sendMessage(CHAT_ID, message, "Markdown");

}

Not finalized yet, save her for further improvisation